ArtBot

Large-Scale Vertical Drawing Machine

Hardware Plan & Architecture Document

IIT Delhi — Abu Dhabi
March 2026

# Table of Contents

1. Project Overview

2. Project Roadmap

3. System Architecture

4. Architecture Comparison (Why Dual-Y Cartesian?)

5. Drive System Comparison

6. Motor Selection

7. Stepper Driver Comparison

8. Pen Lift Mechanism

9. Controller & Firmware

10. Rail & Frame Selection

11. Counterweight System

12. Electronics & Wiring

13. Software Pipeline

14. Bill of Materials

15. Assembly Plan

16. GRBL Configuration

17. Failure Modes & Mitigations

18. References & Resources

# 1. Project Overview

ArtBot is a large-scale robotic drawing machine that converts digital images into pen drawings on a large vertical surface (whiteboard, paper, canvas). Unlike typical CNC plotters that operate on horizontal surfaces, this machine works on a vertical plane — meaning the mechanical design must account for gravity acting on all moving components.

## Key Specifications

| Parameter | Value |
| --- | --- |
| Board Size | 180 cm × 120 cm (vertical) |
| Buffer Zone | 5 cm on all sides |
| Usable Drawing Area | 170 cm × 110 cm |
| Origin | Top-left corner |
| Motion System | Dual-Y Cartesian with counterweights |
| Motors | 3× NEMA 17 steppers + 1× SG90 servo |
| Controller | Arduino Uno + CNC Shield V3 + GRBL firmware |
| Drivers | 3× TMC2209 |
| Resolution | 80 steps/mm (0.0125 mm per step) |
| Pen Tip Width | 0.5–1.0 mm (40–80× larger than step size) |

## The Three Axes

| Axis | Function | Mechanism |
| --- | --- | --- |
| X | Horizontal (left–right) | Belt + motor on rail |
| Y | Vertical (up–down) | Dual motors + belts + counterweights |
| Z | Pen up/down | SG90 servo + compression spring |

# 2. Project Roadmap

ArtBot is being developed in phases. The current focus is Phase 1: getting the machine to produce clean black-and-white line drawings with a single pen. Multi-color pen switching and advanced output techniques are planned for future phases.

Our architecture choices (Dual-Y Cartesian, rigid rails, precise homing) were made with all three phases in mind — so we won't have to rebuild the machine when we add capabilities.

## Phase 1 — Black & White Sketching (Current)

Single black pen (marker, felt-tip, or ballpoint)

Mechanical skeleton: frame, rails, motors, counterweights

Electronics: Arduino + CNC Shield + GRBL + TMC2209 drivers

Software pipeline: image to edge detection to contours to G-Code

Calibration: homing, steps/mm tuning, test square verification

Goal: produce clean, recognizable line drawings on the board

## Phase 2 — Multi-Color (Future)

Pen dock mechanism at top of board (row of pen holders at known coordinates)

Automatic pen switching: carriage moves to dock, drops current pen, picks up next color

Color layer separation in software (split image into color channels → separate G-Code passes)

Registration accuracy: machine must return to exact coordinates between pen changes

This is why we chose rigid rails over cables — cables can't guarantee return precision

## Phase 3 — Advanced Output (Future)

Fill patterns and hatching (parallel lines at varying density for shading)

Halftoning (dots of varying size/spacing to simulate grayscale)

Variable pen pressure via Z-axis stepper upgrade (replaces servo)

Raspberry Pi as wireless G-Code sender (web UI, file storage, WiFi control)

Integration with vpype (plotter-specific path optimization library)

Everything in this document focuses on Phase 1. Phases 2 and 3 are mentioned only to explain why certain architectural decisions were made (e.g., choosing rigid rails over cables for future pen-switch registration).

# 3. System Architecture

The diagram below shows the complete machine layout as seen from the front. Two Y-motors at the top corners move the horizontal X-rail up and down. A third motor on the X-rail moves the pen carriage left and right. Counterweights behind the board balance the X-rail's weight.

# 4. Architecture Comparison

We evaluated four motion architectures for a 180×120 cm vertical plotter. The diagram below shows all four side by side, followed by a detailed comparison.

## A: Polargraph / V-Plotter — REJECTED

Two motors at top corners suspend a pen gondola on cables. Used by: Makelangelo, Maslow CNC, Penelope, Stringent.

Why rejected:

Accuracy collapses at bottom corners — the Jacobian of the coordinate transform shows resolution drops dramatically below the motor line. Most builders recommend 15% margins, wasting ~30% of our board.

Gondola swings on fast direction changes — must draw very slowly.

Cable stretch accumulates over long drawings, causing drift.

Multi-color pen switching is nearly impossible (gondola sways during docking).

Penelope's builder reported accuracy issues 'especially near the bottom and on curves.'

## B: CoreXY / H-Bot — REJECTED

Two stationary motors drive a single continuous belt loop. Used by: most modern 3D printers (Voron, Bambu Lab), Stefan van Seggelen's large plotter.

At 180×120 cm, total belt path is ~12 meters — belt stretch is unmanageable with standard GT2.

Stefan van Seggelen's build required 15mm steel-core GT2 belts (expensive, hard to source).

Getting two 6-meter belt paths tensioned identically is nearly impossible for a first build.

H-Bot variant introduces rotational torque on diagonal moves, causing skew.

## C: Single-Y Cartesian — REJECTED

One motor per axis — simplest possible setup.

Single motor driving one side of a 180 cm rail causes severe racking (rail tilts).

No large-format CNC machine uses single-Y drive — it simply doesn't work at this scale.

## D: Dual-Y Cartesian — CHOSEN

Two synchronized Y-motors (one per side) move the X-rail vertically. A third motor drives the pen carriage horizontally. Counterweights balance gravity. Used by: iDraw H (commercial, ±0.02mm precision), OpenBuilds ACRO, every large CNC router.

Anti-racking: both sides of the rail move simultaneously, keeping it perfectly level.

Short belt runs (~260 cm each) — minimal stretch compared to CoreXY's 12m.

Counterweight-friendly: symmetric design enables easy balancing.

Pen-switch ready: rigid rail means precise return to coordinates for multi-color.

GRBL natively supports dual-Y axis with the CNC Shield's A-axis clone jumper.

Proven at scale by commercial and DIY machines worldwide.

## Architecture Decision Matrix

| Criteria | Polargraph | CoreXY | Single-Y | Dual-Y (Ours) |
| --- | --- | --- | --- | --- |
| Precision at corners | Poor | Good | Poor | Excellent |
| Scale to 180cm | Easy | Hard | OK | Proven |
| Belt stretch risk | N/A (cables) | Severe | Low | Low |
| Anti-racking | N/A | Good | None | Excellent |
| Multi-color future | Very hard | Possible | Hard | Easy |
| Build complexity | Low | High | Low | Medium |
| Cost | $15–100 | $200+ | $100 | $150–200 |
| Community support | Large | Medium | N/A | Large |

# 5. Drive System Comparison

| Drive Type | Speed | Stretch | Self-Lock (Gravity Safe) | Cost | Verdict |
| --- | --- | --- | --- | --- | --- |
| GT2 Timing Belt | Fast (5000 mm/min) | ~0.2% | No — needs counterweight | $5–10 / 10m | CHOSEN |
| Lead Screw (T8) | Slow (500 mm/min) | ~0 | Yes — self-locking | $20–30 / 120cm | Rejected (too slow) |
| Ball Screw | Fast | ~0 | No | $100–300 / 120cm | Rejected (too expensive) |
| Cable / Fishing Line | Medium | High | No | $1 | Rejected (stretch) |
| Beaded Chain | Medium | Low | No | $5–10 | Rejected (backlash) |
| Rack & Pinion | Fast | ~0 | No | $50+ / 180cm | Rejected (cost) |

GT2 timing belt wins on speed, cost, and community support. The self-locking disadvantage is solved by counterweights — when balanced, the rail stays in place even with power off.

# 6. Motor Selection

| Motor | Frame Size | Torque | Weight | Current | Voltage | Cost | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NEMA 14 | 35mm | 10–20 Ncm | ~150g | 0.5–1A | 12V | $5–8 | Too weak |
| NEMA 17 | 42mm | 40–65 Ncm | ~280g | 1.2–2A | 12V | $8–15 | CHOSEN |
| NEMA 23 | 57mm | 100–300 Ncm | ~600g | 2–4A | 24V | $20–40 | Overkill |
| Closed-Loop Servo | Varies | High | ~400g | Varies | 24V+ | $50+ | Too expensive |

With counterweights eliminating gravity load, NEMA 17 (>45 Ncm) provides more than enough torque to position the rail. NEMA 23 would add unnecessary weight to the carriage and require 24V power and higher-current drivers.

# 7. Stepper Driver Comparison

| Driver | Max Current | Microstepping | Noise | Cost | Verdict |
| --- | --- | --- | --- | --- | --- |
| A4988 | 2A (1A safe) | 1/16 | Very loud | $1–2 | Rejected (noise) |
| DRV8825 | 2.2A (1.5A safe) | 1/32 | Loud | $2–3 | Rejected (noise) |
| TMC2209 | 2.8A RMS | 1/256 | Near silent | $3–5 | CHOSEN |

TMC2209's StealthChop mode makes motor operation nearly silent — critical for a machine that may run 30+ minutes per drawing. The extra $1–2 per driver is a trivial cost. Drop-in compatible with CNC Shield V3 (same pinout as A4988).

# 8. Pen Lift Mechanism

On a vertical board, gravity pulls the pen AWAY from the surface. A compression spring pushes the pen toward the board. The servo retracts the pen when traveling between strokes.

| Mechanism | Weight | Speed | Power | Cost | Verdict |
| --- | --- | --- | --- | --- | --- |
| SG90 Servo | 9g | 0.1s / 60° | 5V (from Arduino) | $1–2 | CHOSEN |
| Solenoid | 30–80g | 5–10ms | 12V continuous | $5–10 | Rejected (thumps board) |
| Stepper Motor | 200g+ | Variable | Needs extra driver | $10+ | Rejected (too heavy) |

# 9. Controller & Firmware

| Option | Real-Time? | Dual-Y Support | WiFi | Complexity | Verdict |
| --- | --- | --- | --- | --- | --- |
| Arduino Uno + GRBL | Yes | Native (A-axis clone) | No | Low | CHOSEN (v1) |
| Arduino Mega + Marlin | Yes | Yes | No | Medium | Overkill (3D printer firmware) |
| ESP32 + FluidNC | Yes | Yes | Yes | Medium | Future upgrade path |
| Raspberry Pi (alone) | No | N/A | Yes | High | Rejected (not real-time) |

GRBL on Arduino Uno is the proven standard for 3-axis CNC. It generates motor pulses with microsecond precision and has a look-ahead algorithm that smooths curves automatically. A Raspberry Pi can be added later as a wireless G-Code sender, but cannot replace the Arduino for real-time motor control.

# 10. Rail & Frame Selection

| Profile | Cross Section | Stiffness (Iy) | Weight/m | Deflection at 180cm, 1kg | Use |
| --- | --- | --- | --- | --- | --- |
| V-Slot 2020 | 20×20 mm | ~0.7 cm⁴ | ~0.5 kg | ~4 mm (UNACCEPTABLE) | Y-rails, frame |
| V-Slot 2040 | 20×40 mm | ~3.3 cm⁴ | ~0.9 kg | ~0.9 mm (acceptable) | X-RAIL (chosen) |
| V-Slot 4040 | 40×40 mm | ~7.0 cm⁴ | ~1.6 kg | ~0.4 mm (excellent) | Not needed |

The X-rail (180 cm horizontal span) must be 2040 or stiffer. A 2020 rail will visibly bow at the center. Adding a flat aluminum backing strip (3mm × 30mm × 180cm) to the 2040 provides additional rigidity at almost no cost.

# 11. Counterweight System

## Balancing Procedure

1. Fully assemble the X-rail: rail + carriage + X-motor + servo + pen holder + all brackets.

2. Weigh the complete assembly on a kitchen scale.

3. Start with counterweight at ~95% of that mass (slightly under-balanced).

4. Hang counterweights behind the board via cables routed over top pulleys.

5. The One-Finger Test: push the rail with one finger. It should move smoothly and stay exactly where you leave it.

6. If the rail drifts down: add weight. If it drifts up: remove weight.

7. Once balanced, secure the counterweights so they cannot shift.

# 12. Electronics & Wiring

## Critical Wiring Note — Dual Y-Motors

WARNING: The two Y-motors face opposite directions physically. If you clone them with identical signals (via the A-axis jumper), one will push UP while the other pushes DOWN. The rail will jam and potentially break.

Fix: On ONE of the two Y-motors, swap the wiring of one coil pair. If the motor connector has 4 wires [A+, A−, B+, B−], swap A+ and A− on that one motor. This reverses its rotation, making both motors move the rail in the same direction.

# 13. Software Pipeline

The software converts a digital image into G-Code instructions that the Arduino executes.

The pipeline is implemented in Python and is fully functional. Three CLI tools are provided:

convert.py — converts an image to .gcode (the main pipeline)

simulate.py — visualizes the G-Code as an animated pen plotter preview

tune.py — interactive slider interface for tuning edge detection parameters

# 14. Bill of Materials

## Structural

| Item | Spec | Qty | Purpose |
| --- | --- | --- | --- |
| V-Slot 2040 Extrusion | 180 cm | 1 | X-axis horizontal rail |
| V-Slot 2020 Extrusion | 120 cm | 2 | Y-axis vertical rails |
| V-Slot 2020 Extrusion | 180 cm | 2 | Frame top & bottom |
| 90° Angle Brackets | V-Slot compatible | 8 | Corner joints |
| T-Nuts + M5×8 Bolts | M5 | 50+ pack | Securing components |
| Flat Aluminum Bar | 3mm × 30mm × 180cm | 1 | X-rail stiffener |

## Motion

| Item | Spec | Qty | Purpose |
| --- | --- | --- | --- |
| GT2 Timing Belt | 6mm wide, open-ended | 10m roll | All axis drive |
| GT2 Pulley (20-tooth) | 5mm bore | 3 | Motor shafts |
| GT2 Idler Pulley | 5mm bore | 5 | Belt routing |
| V-Slot Gantry Plate | Standard OpenBuilds | 3 | Rail carriages |
| Mini V-Wheels (Delrin) | Standard | 12 | 4 per gantry plate |
| Eccentric Spacers | Standard | 6 | Wheel tension adjust |
| Belt Tensioners | Adjustable, spring-loaded | 3 | One per belt run |

## Motors & Electronics

| Item | Spec | Qty | Purpose |
| --- | --- | --- | --- |
| NEMA 17 Stepper Motor | >45 Ncm, 1.5–2A | 3 | X + Y-Left + Y-Right |
| SG90 Micro Servo | Standard | 1 | Pen lift (Z-axis) |
| Motor Mounting Bracket | NEMA 17 L-bracket | 3 | Motor mounting |
| Arduino Uno R3 | Original or clone | 1 | GRBL controller |
| CNC Shield V3 | Fits Arduino Uno | 1 | Driver interface |
| TMC2209 Driver Module | StepStick form factor | 3 | Motor drivers |
| 12V 10A Power Supply | Mean Well or equiv. | 1 | Main power |
| Limit Switches | NO microswitch, lever | 3 | Homing (X, Y, spare) |
| Drag Chain | 10×10mm flexible | 2m | Cable management |
| USB-B Cable | Standard Arduino | 1 | PC connection |

## Counterweight & Pen

| Item | Spec | Qty | Purpose |
| --- | --- | --- | --- |
| Nylon Cord / Paracord | 3mm, rated >20kg | 5m | Counterweight cables |
| Small Bearing Pulleys | 20–30mm diameter | 4 | Cable routing (top) |
| Counterweight Mass | Steel / sand / water | As needed | Balance X-rail |
| Compression Spring | ~10mm OD, ~20mm | 1 | Push pen toward board |
| Pen Holder | 3D-printed or purchased | 1 | Holds marker / pen |

# 15. Assembly Plan

## Phase 1: Frame (Day 1–2)

Lay out all 4 frame extrusions on the floor.

Bolt corners with angle brackets + T-nuts.

CRITICAL: Measure BOTH diagonals — they must be equal (within 2mm) or drawings will be skewed.

Mount frame to drawing surface or backing structure with at least 6 mounting points.

## Phase 2: Y-Axis (Day 3–4)

Install V-wheels on the two Y-axis gantry plates.

Slide gantry plates onto left and right vertical rails.

Adjust eccentric spacers: wheels should grip snugly but roll freely.

Mount Y-Left motor at top-left, Y-Right motor at top-right.

Install idler pulleys at the bottom of each vertical rail.

Route Y-axis belts: motor → down to idler → back up to gantry plate.

Test: turn both motor shafts BY HAND — both gantry plates should move smoothly.

## Phase 3: X-Axis (Day 5–6)

Bolt the 180cm 2040 rail between the two Y-axis gantry plates.

Check level — the X-rail must be perfectly horizontal.

Install pen carriage on the X-rail with V-wheels.

Mount X-motor at the left end, idler pulley at the right end.

Route X-axis belt and install belt tensioner.

Optional: bolt mid-span aluminum stiffener to back of X-rail.

## Phase 4: Counterweights (Day 6–7)

Weigh the entire X-rail assembly.

Install pulleys at the top of the frame behind the board.

Attach cord to both ends of the X-rail, route over pulleys, hang weights.

Perform the One-Finger Test (see Section 10).

## Phase 5: Electronics (Day 7–8)

Flash GRBL firmware onto the Arduino Uno.

Plug CNC Shield onto Arduino, insert TMC2209 drivers.

Set A-axis jumper on CNC Shield to clone Y.

Wire motors: X→X slot, Y-Left→Y slot, Y-Right→A slot.

REVERSE ONE Y-MOTOR'S COIL (swap A+ and A− on one motor).

Wire servo to SpnEn pin.

Wire and mount limit switches (X-min, Y-min).

Connect 12V 10A PSU.

## Phase 6: Calibration (Day 9–10)

Connect to GRBL via Universal Gcode Sender.

Configure GRBL $ parameters (see Section 15).

Home the machine ($H) — should move to top-left and stop at switches.

Test X and Y directions with small jog commands.

Draw a test square and measure with a ruler.

If dimensions are wrong, adjust $100 / $101 (steps/mm).

# 16. GRBL Configuration

Enter these via the serial console in Universal Gcode Sender:

| Parameter | Value | Description |
| --- | --- | --- |
| $100 | 80 | X steps/mm (200 steps × 16 microsteps / 20 teeth × 2mm) |
| $101 | 80 | Y steps/mm (same calculation) |
| $110 | 5000 | X max feed rate (mm/min) |
| $111 | 5000 | Y max feed rate (mm/min) |
| $120 | 200 | X acceleration (mm/s²) — start low, increase gradually |
| $121 | 100 | Y acceleration (mm/s²) — lower than X (heavy rail) |
| $130 | 1700 | X max travel (mm) |
| $131 | 1100 | Y max travel (mm) |
| $22 | 1 | Enable homing cycle |
| $23 | 3 | Homing direction invert (both X and Y home to min) |
| $24 | 25 | Homing feed rate (mm/min) — slow approach to switch |
| $25 | 500 | Homing seek rate (mm/min) — fast initial move |

## Test Square G-Code

Send this to verify calibration. Draws a 150mm square — measure with a ruler:

G21           ; Units: mm
G90           ; Absolute positioning
G0 X50 Y50 F5000    ; Move to start
M3 S90        ; Pen down
G4 P0.15      ; Wait for pen settle
G1 X200 Y50 F3000   ; Draw right
G1 X200 Y200        ; Draw down
G1 X50 Y200         ; Draw left
G1 X50 Y50          ; Draw up (close square)
M5            ; Pen up
G0 X0 Y0 F5000      ; Return home
M2            ; Program end

# 17. Failure Modes & Mitigations

| Failure | Cause | Prevention |
| --- | --- | --- |
| Rail crashes on power loss | No counterweight / belt breaks | Counterweights hold rail in place |
| Skipped steps (drawing shifts) | Accel too high, PSU weak, belt loose | Tune $120/$121, use 10A PSU, tension belts |
| Y-axis jams on startup | Y-motors fighting each other | Reverse one motor's coil wiring |
| Drawing is skewed (rhombus) | Frame not square | Measure diagonals during assembly |
| Pen doesn't touch board | Spring too weak / servo miscalibrated | Tune spring, calibrate servo angle |
| Belt skips teeth | Belt loose, pulley grub screw loose | Belt tensioners + Loctite on grub screws |
| X-rail bows at center | 180cm 2020 profile too flexible | Use 2040 + mid-span stiffener |
| Lines are wavy/vibrating | Rail chatter at high speed | Reduce feed rate, stiffen rail, lower accel |
| Homing fails | Switch not triggering / wrong config | Test switches manually, check $22/$23 |

# 18. References & Resources

## Real-World Builds & Projects

Makelangelo (Polargraph) — github.com/MarginallyClever/Makelangelo-software

Maslow CNC (Wall-Mounted Router) — maslowcnc.com

Penelope Vertical Pen Plotter — davidbliss.com/2021/09/13/penelope

Stefan van Seggelen Large CoreXY Plotter — stefanvanseggelen.com/plotter.html

iDraw H Large Format Plotter — uunatek.com/collections/large-format-size-drawing-robots

OpenBuilds Pen Plotter — builds.openbuilds.com/builds/pen-plotter.10615

OpenBuilds ACRO System — builds.openbuilds.com/tags/acro

Jason Webb's GRBL Wall Plotter — github.com/jasonwebb/grbl-mega-wall-plotter

Stringent $15 Wall Plotter — hackster.io/fredrikstridsman/stringent-the-15-wall-plotter-d965ca

## Technical References

GRBL Setup Guide — howtomechatronics.com/tutorials/how-to-setup-grbl-control-cnc-machine-with-arduino

GRBL with CNC Shield — diyengineers.com/2023/01/05/grbl-with-arduino-cnc-shield-complete-guide

CoreXY Belt Tensioning — drmrehorst.blogspot.com/2018/08/corexy-mechanism-layout-and-belt.html

Belts vs Leadscrews in CNC — blanch.org/belts-vs-screws-in-cnc-design

V-Slot Deflection Calculator — builds.openbuilds.com/threads/how-to-calculate-v-slot-deflection.4881

NEMA 17 vs NEMA 23 — sss-motors.com/nema-17-vs-nema-23-stepper-motor

TMC2209 vs TMC2208 — xecor.com/blog/tmc2208-vs-tmc2209

Polargraph Mathematics — nexp.pt/vbot.html

FluidNC (Modern GRBL Replacement) — wiki.fluidnc.com/en/config/kinematics